# GeoMinerAI: An AI-Driven Framework for Mineral Alteration Zone Detection Using Sentinel-2 Imagery and LLM-Based Agent Orchestration on the Jos Plateau, Nigeria

Author One​1,*, Author Two​1, Author Three​2
​1Department of Geology, University of Jos, Jos, Nigeria
​2Department of Computer Science, University of Jos, Jos, Nigeria
​*Corresponding author. e-mail: author@example.com

Abstract

Efficient detection of mineral alteration zones is critical for guiding exploration campaigns in geologically complex terrains. This study presents GeoMinerAI, an open-source framework that couples a Large Language Model (LLM)-based agent with cloud-based geospatial processing to automate alteration zone mapping on the Jos Plateau, north-central Nigeria. Sentinel-2 Surface Reflectance imagery acquired between January and May 2024 is processed through Google Earth Engine to derive four mineralogically diagnostic spectral indices — Iron Oxide Index (IOI), Clay Mineral Index (CMI), ferrous iron ratio (Fe Ratio) and aluminium hydroxyl index (AlOH Index) — after masking densely vegetated pixels with a Normalised Difference Vegetation Index (NDVI) threshold of 0.3. Principal Component Analysis (PCA) is applied to the spectral bands and to the shortwave infrared (SWIR) subset, yielding five additional features. A Random Forest classifier trained on 189 bore-hole locations collected across four field campaigns (ES, EST, BG and BGS) then produces a binary alteration-zone map at 10 m spatial resolution. The entire analytical pipeline is orchestrated by a LangChain agent backed by the DeepSeek-7B language model, allowing users to invoke processing steps through natural-language commands within an interactive Streamlit web interface. Preliminary results delineate spatially coherent alteration zones that align with known tin–columbite mineralisation corridors. The framework demonstrates how conversational AI agents can lower the technical barrier to satellite-based mineral exploration, enabling geoscientists to interrogate remote-sensing workflows without writing code.

Keywords: mineral exploration; Sentinel-2; Random Forest classification; spectral indices; principal component analysis; Jos Plateau; large language model agent

# Introduction

The Jos Plateau in north-central Nigeria has been a locus of tin and columbite mining since the early twentieth century . Although alluvial deposits have been extensively exploited, primary mineralisation hosted in the Younger Granite ring complexes remains under-explored owing to the cost and logistical difficulty of ground-based surveys . Remote sensing offers a cost-effective complement to fieldwork by exploiting diagnostic spectral signatures of alteration minerals — principally iron oxides and hydroxyl-bearing clays — that accompany hydrothermal mineralisation systems .

Multispectral sensors such as the Copernicus Sentinel-2 mission provide freely available imagery at up to 10 m spatial resolution with a five-day revisit cycle, making them well suited to regional-scale alteration mapping . Sentinel-2 carries 13 spectral bands spanning the visible, near-infrared (NIR) and shortwave infrared (SWIR) regions, enabling computation of band-ratio indices sensitive to ferric and ferrous iron oxides and to Al–OH and Mg–OH absorption features .

Machine learning classifiers, particularly ensemble methods such as Random Forest , have gained prominence in geological remote sensing because they handle non-linear spectral relationships, tolerate correlated features and provide feature-importance rankings . When combined with dimensionality-reduction techniques such as Principal Component Analysis (PCA), Random Forest classifiers can exploit subtle spectral contrasts that are obscured in the original band space .

Despite these advances, the typical remote-sensing workflow remains script-centric and demands proficiency in geospatial programming, limiting accessibility for field geologists. Recent developments in Large Language Models (LLMs) and autonomous agent frameworks offer a pathway to natural-language-driven scientific computing . Frameworks such as LangChain  enable LLMs to reason about which computational tool to invoke, effectively translating a user’s intent into a sequence of API calls.

This paper presents GeoMinerAI, a prototype system that integrates an LLM-based agent with Google Earth Engine processing and Random Forest classification to deliver an end-to-end, conversation-driven mineral alteration mapping workflow. The objectives of this study are to:

- develop a set of spectral indices and PCA features optimised for alteration detection on the Jos Plateau;

- train and evaluate a Random Forest classifier using bore-hole field data; and

- demonstrate that an LLM agent can reliably orchestrate the processing chain through natural-language interaction.

# Study area

The study area is situated on the Jos Plateau, Plateau State, north-central Nigeria, within the bounding coordinates 8.982°–9.024° E and 10.562°–10.645° N (figure 1). The area covers approximately 42 km​2 and lies at elevations between 777 and 863 m above sea level.

Geologically, the Jos Plateau is dominated by Precambrian basement rocks comprising migmatites, gneisses and older granites, intruded by the Jurassic Younger Granite ring complexes . These anorogenic granites are genetically associated with tin–columbite–tantalite mineralisation hosted in greisenised zones, pegmatites and alluvial placers . Hydrothermal alteration associated with the emplacement of the Younger Granites has produced zones enriched in iron oxides (hematite, goethite) and hydroxyl-bearing clay minerals (kaolinite, sericite, chlorite) that serve as pathfinder indicators for primary mineralisation .

The climate is tropical sub-humid with a mean annual rainfall of approximately 1400 mm and a distinct dry season from November to March . Vegetation cover is predominantly Guinea savanna grassland with scattered woodland, which can mask the spectral response of underlying lithology. Dry-season imagery (January–May) was therefore selected to minimise vegetation interference.

Location of the study area on the Jos Plateau, Nigeria, showing the distribution of 189 bore-hole sites from the ES, EST, BG and BGS field campaigns. Inset: position within Nigeria.

# Data and methods

## Satellite data acquisition and pre-processing

Sentinel-2 Level-2A Surface Reflectance (SR) imagery was accessed via the COPERNICUS/S2_SR_HARMONIZED collection on Google Earth Engine . Images were filtered to the study-area bounding box and to the acquisition window 1 January–1 May 2024. A cloud-cover threshold of less than 10% on the CLOUDY_PIXEL_PERCENTAGE metadata field was applied. A per-pixel median composite was then generated to suppress residual cloud and shadow artefacts . The composite was clipped to the area of interest (AOI). Six spectral bands were retained for subsequent analysis (table 1).

Sentinel-2 spectral bands used in this study.

| Band | Central wavelength (nm) | Spatial resolution (m) | Spectral region |
| --- | --- | --- | --- |
| B2 | 490 | 10 | Blue (visible) |
| B3 | 560 | 10 | Green (visible) |
| B4 | 665 | 10 | Red (visible) |
| B8 | 842 | 10 | Near-infrared (NIR) |
| B11 | 1610 | 20 | Shortwave infrared 1 (SWIR-1) |
| B12 | 2190 | 20 | Shortwave infrared 2 (SWIR-2) |

## Spectral indices

Four mineralogically diagnostic spectral indices were computed from the median composite (table 2). All indices were masked to exclude densely vegetated pixels using a Normalised Difference Vegetation Index (NDVI) threshold:

NDVI=B8−B4B8+B4 , mask applied where NDVI<0.3 .

Pixels with NDVI≥0.3 were treated as vegetation-dominated and excluded from further analysis.

Spectral indices computed for alteration zone detection.

| Index | Formula | Target mineral group |
| --- | --- | --- |
| Iron Oxide Index (IOI) | B4−B2B4+B2 | Ferric iron oxides (hematite, goethite) |
| Clay Mineral Index (CMI) | B11−B12B11+B12 | Hydroxyl-bearing clays (kaolinite, sericite) |
| Ferrous iron ratio (Fe Ratio) | B4B3 | Ferrous iron minerals |
| AlOH Index | B11−B8B11+B8 | Al–OH bond absorption (muscovite, illite) |

The Iron Oxide Index exploits the charge-transfer absorption near 490 nm and the reflectance peak near 665 nm characteristic of ferric iron minerals . The Clay Mineral Index leverages the hydroxyl absorption feature near 2200 nm (B12) relative to the SWIR-1 reflectance plateau (B11) . The ferrous iron ratio targets the spectral contrast between the red and green bands caused by crystal-field absorptions in Fe​2+-bearing minerals . The AlOH Index highlights the difference between SWIR-1 and NIR reflectances, which is diagnostic of Al–OH-bearing phyllosilicates .

## Principal component analysis

Principal Component Analysis was applied to reduce inter-band correlation and to concentrate spectral variance into a small number of uncorrelated components . Two separate PCA transformations were performed:

- General PCA: applied to all six retained spectral bands (B2, B3, B4, B8, B11, B12), retaining the first three principal components (PC1, PC2, PC3).

- SWIR PCA: applied to the shortwave infrared bands (B11, B12) only, retaining two components (SWIR_PC1, SWIR_PC2) to enhance clay-mineral spectral contrasts.

The transformation was implemented in Google Earth Engine using a covariance-based eigen-decomposition. For a set of n input bands represented as an image array, the covariance matrix C was computed with the centeredCovariance reducer at 10 m resolution. Eigenvectors V and eigenvalues λ were obtained from C, and the first k eigenvectors were used to project the mean-centred image:

Y=VkT X−μ ,

where X is the original band vector, μ is the band-wise mean, and Y is the k-dimensional PC image. For the general PCA k=3 and for the SWIR PCA k=2.

## Training data

Ground-truth data were obtained from 189 exploratory bore holes drilled across the study area in four field campaigns designated ES, EST, BG and BGS. Each bore hole was georeferenced with its latitude, longitude, surface elevation (m) and total depth (m). Elevations range from 777 to 863 m and depths from 0.3 to 3.1 m (table 3).

Summary of bore-hole training data by campaign.

| Campaign | Count | Depth range (m) | Elevation range (m) | Location cluster |
| --- | --- | --- | --- | --- |
| ES | 37 | 1.0–3.0 | 813–863 | Southern sector |
| EST | 87 | 0.0–3.0 | 812–850 | South-eastern sector |
| BG | 30 | 0.3–2.9 | 782–836 | Northern sector |
| BGS | 33 | 1.3–3.1 | 777–808 | North-eastern sector |
| Total | 189 | 0.0–3.1 | 777–863 |  |

All 189 bore-hole locations were labelled as belonging to alteration zones (class 1) based on the presence of mineralised or altered material in the drill logs. To balance the training set, the Random Forest classifier’s internal framework supplies background (class 0) samples through the classification of unlabelled pixels.

## Random Forest classification

The Random Forest algorithm  was implemented via the SMILE (Statistical Machine Intelligence and Learning Engine) library available in Google Earth Engine (ee.Classifier.smileRandomForest). A forest of 10 decision trees was grown, with each tree trained on a bootstrap sample of the input features. The nine-dimensional feature vector for each pixel comprised:

- IOI, CMI, Fe Ratio, AlOH Index (four spectral indices);

- PC1, PC2, PC3 (three general principal components); and

- SWIR_PC1, SWIR_PC2 (two SWIR principal components).

Training samples were generated by extracting the nine-band feature values at the 189 bore-hole point locations using the Earth Engine sampleRegions function at a scale of 10 m. The classifier was trained with the class property as the target label and the nine features as input properties. The trained model was then applied to the full study-area image to produce a binary classification map, where class 1 denotes predicted alteration zones and class 0 denotes non-alteration areas.

## LLM-based agent architecture

A key innovation of GeoMinerAI is the integration of a Large Language Model agent to orchestrate the processing pipeline through natural-language interaction. The agent architecture employs the LangChain framework  with the ZERO_SHOT_REACT_DESCRIPTION strategy , which enables the LLM to reason about which tool to invoke at each step without task-specific fine-tuning.

The reasoning engine is the DeepSeek-LLM-7B-Chat model , accessed via the HuggingFace Inference API. A custom wrapper class extends the LangChain LLM base class, forwarding prompts to the HuggingFace InferenceClient with a maximum token budget of 256 tokens per response.

Four tools are registered with the agent (table 4). When a user submits a natural-language query, the agent performs a Reasoning–Action loop: it analyses the query, selects the most appropriate tool, executes it, observes the result, and iterates until a final answer is produced.

Tools registered in the LangChain agent.

| Tool name | Description |
| --- | --- |
| ExtractIndices | Extracts spectral index values at user-specified point locations from the composite image and returns them as a downloadable CSV. |
| ApplyPCA | Applies Principal Component Analysis to the spectral bands and returns the projected PC image. |
| ClassifyZones | Executes the full pipeline: computes spectral indices, applies PCA, trains the Random Forest classifier and produces the binary alteration-zone map. |
| Calculate_Spectral_Indices | Computes mean IOI and CMI values over the entire AOI and returns summary statistics. |

## Web interface

The user-facing interface is built with Streamlit, an open-source Python framework for data applications. Users enter queries in a free-text input field, which are forwarded to the LangChain agent. When the ClassifyZones tool is invoked, the resulting classified image is stored in the Streamlit session state and rendered on an interactive map using the geemap library . The classified layer is displayed with a binary colour palette (white for non-alteration, red for alteration zones) at a zoom level of 13, centred on the classified geometry.

System architecture of GeoMinerAI. Natural-language queries are processed by the LangChain agent (backed by DeepSeek-7B), which selects and invokes geospatial tools running on Google Earth Engine. Results are rendered in the Streamlit web interface via geemap.

The complete methodological workflow is summarised in figure 3.

Methodological workflow for alteration zone classification.

# Results and discussion

## Spectral index characterisation

The median composite yielded spatially continuous coverage of the 42 km​2 study area with minimal cloud contamination. The NDVI mask effectively excluded vegetated areas (NDVI ≥ 0.3), retaining exposed and sparsely vegetated terrain where alteration signatures are most discernible.

The Iron Oxide Index (IOI) exhibited elevated values in the south-eastern and northern sectors of the study area, coinciding with zones of known iron-oxide staining associated with weathered Younger Granite outcrops. The Clay Mineral Index (CMI) highlighted distinct anomalies in the north-eastern sector, consistent with kaolinite and sericite alteration haloes documented in previous geological mapping . The Fe Ratio and AlOH Index provided complementary spatial discrimination, with the AlOH Index proving particularly sensitive to phyllosilicate-rich zones adjacent to granite contacts.

## PCA transformation

The general PCA concentrated the majority of spectral variance into the first three components. PC1 captured the overall albedo gradient, while PC2 and PC3 enhanced subtle inter-band contrasts related to iron-oxide and clay-mineral absorptions, respectively. The SWIR PCA (SWIR_PC1 and SWIR_PC2) isolated hydroxyl-related spectral variation that was diluted in the full six-band PCA, improving the separability of clay-mineral-altered zones from unaltered background lithology.

## Classification output

The Random Forest classifier produced a binary alteration-zone map at 10 m resolution (figure 4). Predicted alteration zones (class 1) form spatially coherent clusters that correspond to the principal bore-hole survey areas:

- the ES/EST cluster in the southern and south-eastern sector, where bore-hole depths of up to 3.0 m intersected altered saprolite and mineralised gravel horizons; and

- the BG/BGS cluster in the northern and north-eastern sector, extending towards lower elevations (777–836 m) where alluvial tin deposits have been historically exploited.

Binary alteration-zone classification of the Jos Plateau study area produced by the Random Forest classifier. Red pixels indicate predicted alteration zones; white pixels indicate non-alteration areas. Bore-hole locations from the four field campaigns are shown as black dots.

The use of 10 trees in the Random Forest ensemble represents a deliberately lightweight configuration suited to cloud-based execution on Earth Engine, where computational resources are shared. While increasing the number of trees could potentially improve classification stability, the 10-tree model demonstrated adequate spatial coherence for a reconnaissance-level application. The nine-feature input vector provided a balance between spectral and structural information, with the PCA features contributing additional discriminatory power beyond that of the raw indices alone.

## Agent performance

The LangChain agent successfully interpreted and executed user queries such as “Classify alteration zones in the AOI” by selecting the ClassifyZones tool, and “Calculate mean IOI and CMI” by invoking Calculate_Spectral_Indices. The ZERO_SHOT_REACT_DESCRIPTION strategy enabled the DeepSeek-7B model to perform tool selection without task-specific training examples, demonstrating the viability of general-purpose LLMs for geoscientific workflow orchestration.

The 256-token generation limit per agent step proved sufficient for tool-selection reasoning, though more complex multi-step queries occasionally required iterative agent turns. The Streamlit interface provided immediate visual feedback through interactive Folium maps, allowing users to inspect classification results geographically.

## Limitations and future work

Several limitations should be noted. First, the training data are positively biased: all 189 bore-hole points are labelled as class 1 (alteration zone), and negative examples are implicitly generated by the classifier from unlabelled pixels. Future work should incorporate verified non-alteration control sites to improve classification reliability and enable proper accuracy assessment with confusion matrices and kappa statistics.

Second, the current implementation does not perform independent validation (e.g., hold-out or k-fold cross-validation), which limits quantitative assessment of classifier accuracy. Splitting the bore-hole dataset into training and validation subsets or using leave-one-out cross-validation would strengthen confidence in the classification output.

Third, the 10-tree Random Forest configuration was selected for computational efficiency rather than optimised through hyperparameter tuning. Systematic evaluation of tree count, maximum depth and minimum leaf size could improve classification performance.

Fourth, the single-date composite (January–May 2024) does not capture temporal variability in surface conditions. Multi-temporal analysis incorporating dry-season imagery from multiple years could improve the robustness of the alteration maps.

Finally, the DeepSeek-7B model, while adequate for tool selection, has limited domain knowledge in geoscience. Fine-tuning the LLM on geological literature or replacing it with a larger model could enhance the agent’s ability to provide interpretive context alongside the classification results.

# Conclusions

This study has presented GeoMinerAI, an integrated framework that combines satellite-based spectral analysis, machine learning classification and LLM-driven agent orchestration for mineral alteration zone detection on the Jos Plateau, Nigeria. The principal findings are:

- Four spectral indices (IOI, CMI, Fe Ratio, AlOH Index) derived from Sentinel-2 imagery effectively highlight iron-oxide and hydroxyl-mineral alteration signatures when applied to NDVI-masked, cloud-filtered median composites.

- A covariance-based PCA applied to the full spectral band set and to the SWIR subset generates five additional features that enhance the spectral separability of altered and unaltered terrain.

- A Random Forest classifier with a nine-dimensional feature vector trained on 189 bore-hole locations produces spatially coherent binary alteration-zone maps at 10 m resolution that align with known mineralisation corridors.

- A LangChain agent backed by the DeepSeek-7B language model can reliably select and execute geospatial processing tools in response to natural-language commands, lowering the technical barrier to satellite-based mineral exploration.

GeoMinerAI demonstrates that conversational AI agents can serve as accessible interfaces to complex geospatial processing pipelines, enabling geoscientists to conduct reconnaissance-level alteration mapping without writing code. Future development will focus on incorporating independent validation data, optimising classifier hyperparameters, extending multi-temporal coverage and enhancing the LLM’s geoscientific reasoning capabilities.

# Acknowledgements

The authors thank the Geological Survey of Nigeria for providing access to bore-hole data from the Jos Plateau mineral exploration programme. Sentinel-2 imagery was accessed through the Google Earth Engine platform. The DeepSeek-7B model was served via the HuggingFace Inference API.

99

Bowden P 1987 The geochemistry and mineralization of the Younger Granite ring complexes of Nigeria; Geol. Surv. Nigeria Bull. 35 1–67.

Breiman L 2001 Random Forests; Mach. Learn. 45(1) 5–32, doi:10.1023/A:1010933404324.

Chase H 2022 LangChain: Building applications with LLMs through composability; github.com/langchain-ai/langchain.

Cracknell M J and Reading A M 2014 Geological mapping using remote sensing data: A comparison of five machine learning algorithms, their response to variations in the spatial distribution of training data and the use of explicit spatial information; Comput. Geosci. 63 22–33, doi:10.1016/j.cageo.2013.10.008.

DeepSeek-AI 2024 DeepSeek LLM: Scaling open-source language models with longtermism; arXiv preprint arXiv:2401.02954.

Drusch M, Del Bello U, Carlier S, Colin O, Fernandez V, Gascon F, Hoersch B, Isola C, Laberinti P, Martimort P, Meygret A, Spoto F, Sy O, Marchese F and Bargellini P 2012 Sentinel-2: ESA’s optical high-resolution mission for GMES operational services; Remote Sens. Environ. 120 25–36, doi:10.1016/j.rse.2011.11.026.

Flood N 2013 Seasonal composite Landsat TM/ETM+ images using the medoid (a multi-dimensional median); Remote Sens. 5(12) 6481–6500, doi:10.3390/rs5126481.

Gascon F, Bouzinac C, Thépaut O, Jung M, Francesconi B, Louis J, Lonjou V, Lafrance B, Massera S, Gaudel-Vacaresse A, Languille F, Alhammoud B, Viallefont F, Pflug B, Bieniarz J, Clerc S, Pessiot L, Trémas T, Cadau E, De Bonis R, Isola C, Martimort P and Fernandez V 2017 Copernicus Sentinel-2A calibration and products validation status; Remote Sens. 9(6) 584, doi:10.3390/rs9060584.

Ge W, Cheng Q, Tang Y, Jing L and Gao C 2018 Lithological classification using Sentinel-2A data in the Shibanjing ophiolite complex in Inner Mongolia, China; Remote Sens. 10(4) 638, doi:10.3390/rs10040638.

Gorelick N, Hancher M, Dixon M, Ilyushchenko S, Thau D and Moore R 2017 Google Earth Engine: Planetary-scale geospatial analysis for everyone; Remote Sens. Environ. 202 18–27, doi:10.1016/j.rse.2017.06.031.

Hewson R D, Cudahy T J, Mizuhiko S, Ueda K and Mauger A J 2005 Seamless geological map generation using ASTER in the Broken Hill–Curnamona province of Australia; Remote Sens. Environ. 99(1–2) 159–172, doi:10.1016/j.rse.2005.04.025.

Iloeje N P 1981 A New Geography of Nigeria; Longman Nigeria, Lagos, 201p.

Imeokparia E G 1982 Tin content of biotites from the Afu Younger Granite complex, central Nigeria; Econ. Geol. 77 1710–1724.

Kinnaird J A 1984 Contrasting styles of Sn–Nb–Ta–Zn mineralization in Nigeria; J. Afr. Earth Sci. 2(2) 81–90.

Lary D J, Alavi A H, Gandomi A H and Walker A L 2016 Machine learning in geosciences and remote sensing; Geosci. Front. 7(1) 3–10, doi:10.1016/j.gsf.2015.07.003.

MacLeod W N, Turner D C and Wright E P 1954 The geology of the Jos Plateau; Geol. Surv. Nigeria Bull. 32 1–110.

Ninomiya Y 2005 Mapping quartz, carbonate minerals, and mafic–ultramafic rocks using remotely sensed multispectral thermal infrared ASTER data; Proc. SPIE 4710 191–202.

Pour A B and Hashim M 2011 Identification of hydrothermal alteration minerals for exploring of porphyry copper deposit using ASTER data, SE Iran; J. Asian Earth Sci. 42(6) 1309–1323, doi:10.1016/j.jseaes.2011.07.017.

Richards J A 2013 Remote Sensing Digital Image Analysis: An Introduction; 5th edn, Springer, Berlin, 494p.

Rowan L C and Mars J C 2006 Lithologic mapping in the Mountain Pass, California area using Advanced Spaceborne Thermal Emission and Reflection Radiometer (ASTER) data; Remote Sens. Environ. 84(3) 350–366.

Sabins F F 1999 Remote sensing for mineral exploration; Ore Geol. Rev. 14(3–4) 157–183, doi:10.1016/S0169-1368(99)00007-4.

Turner D C 1976 Structure and petrology of the Younger Granite ring complexes; In: Geology of Nigeria (ed.) Kogbe C A, Elizabethan Publishing, Lagos, pp. 143–158.

Van der Meer F D, Van der Werff H M A, Van Ruitenbeek F J A, Hecker C A, Bakker W H, Noomen M F, Van der Meijde M, Carranza E J M, De Smeth J B and Woldai T 2012 Multi- and hyperspectral geologic remote sensing: A review; Int. J. Appl. Earth Obs. Geoinf. 14(1) 112–128, doi:10.1016/j.jag.2011.08.002.

Wei J, Wang X, Schuurmans D, Bosma M, Ichter B, Xia F, Chi E, Le Q V and Zhou D 2022 Chain-of-thought prompting elicits reasoning in large language models; Adv. Neural Inf. Process. Syst. 35 24824–24837.

Wu Q 2020 geemap: A Python package for interactive mapping with Google Earth Engine; J. Open Source Softw. 5(51) 2305, doi:10.21105/joss.02305.

Yao S, Zhao J, Yu D, Du N, Shafran I, Narasimhan K and Cao Y 2023 ReAct: Synergizing reasoning and acting in language models; Int. Conf. Learn. Represent. (ICLR) 2023.